QUALIFICATION NAME: NATIONAL DIPLOMA INFORMATION TECHNOLOGY (COMMUNICATION NETWORKS)

SUBJECT NAME: COMMUNICATION NETWORKS IIB)

SUBJECT CODE: COB30BT

DEPARTMENT NAME: ICT

Student number; Student name and initials. 219618077; Malatjie LC. 219471122; Makalapetlo MC.

Contents

| IP Plan CHIEFS VLAN |  |  |  |
| --- | --- | --- | --- |
| Subnet address | First usable address | Last usable address | Subnet mask |
| 172.30.0.0 | 172.30.0.1 | 172.30.0.254 | 255.255.255.0 |
| PIRATES VLAN |  |  |  |
| Subnet address | First usable address | Last usable address | Subnet mask |
| 172.30.1.0 | 172.30.1.1 | 172.30.1.126 | 255.255.255.128 |
| SWALLOWS VLAN |  |  |  |
| Subnet address | First usable address | Last usable address | Subnet mask |
| 172.30.1.192 | 172.30.1.193 | 172.30.1.254 | 255.255.255.192 |
| SUNDOWNS VLAN |  |  |  |
| Subnet address | First usable address | Last usable address | Subnet mask |
| 172.30.1.128 | 172.30.1.129 | 172.30.1.1.190 | 255.255.255.192 |
| WAN1 |  |  |  |
| Subnet address | First usable address | Last usable address | Subnet mask |
| 172.30.2.0 | 172.30.2.1 | 172.30.2.2 | 255.255.255.252 |
| WAN2 |  |  |  |
| Subnet address | First usable address | Last usable address | Subnet mask |
| 170.30.2.4 | 172.30.2.5 | 172.30.2.6 | 255.255.255.252 |
| WAN3 |  |  |  |
| Subnet address | First usable address | Last usable address | Subnet mask |
| 172.30.2.8 | 172.30.2.9 | 172.30.2.10 | 255.255.255.252 |
| WAN4 |  |  |  |
| Subnet address | First usable address | Last usable address | Subnet mask |
| 172.30.2.12 | 172.30.2.13 | 172.30.2.14 | 255.255.255.252 |

The IP addresses for all devices.

| DEVICE | INTERFACE | IP ADDRESS | SUBNET MASK | DEFAULT GATEWAY |
| --- | --- | --- | --- | --- |
| Router1 | S0/0/0 | 172.30.2.1 | 255.255.255.252 | N/A |
|  | S0/2/0 | 172.30.2.2 | 255.255.255.252 | N/A |
|  | G0/0.10 | 172.30.0.1 | 255.255.255.0 | N/A |
|  | G0/0.20 | 172.30.2.1 | 255.255.255.128 | N/A |
|  | G0/0.30 | 172.30.1.1 | 255.255.255.192 | N/A |
|  | G0/0.40 | 172.30.1.129 | 255.255.255.192 | N/A |
| Router 2 | G0/0 | 100.10.10.1 | 255.255.255.248 | N/A |
|  | S0/1/1 | 172.30.2.5 | 255.255.255.252 | N/A |
|  | S0/2/0 | 172.30.2.6 | 255.255.255.252 | N/A |
| Router 3 | S0/0/0 | 172.30.2.9 | 255.255.255.252 | N/A |
|  | S0/0/1 | 172.30.2.10 | 255.255.255.255 | N/A |
|  |  |  |  | N/A |
| Router 4 | G0/0 | 30.30.30.1 | 255.255.255.128 | N/A |
|  | S0/0/1 | 172.30.2.13 | 255.255.255.252 | N/A |
|  | S0/1/1 | 172.30.2.13 | 255.255.255.252 | N/A |
| Wireless Router CHIEFS | NIC | 172.30.0.1 | 255.255.255.0 | N/A |
| Wireless Router PIRATES | NIC | 172.30.1.1 | 255.255.255.128 | N/A |
| Wireless Router SUNDOWNS | NIC | 172.30.1.129 | 255.255.255.192 | N/A |
| Wireless Router SWALLOWS | NIC | 172.30.1.192 | 255.255.255.192 | N/A |
| PC0 (CHIEFS) | NIC | 172.30.0.3 | 255.255.255.0 | 172.30.0.1 |
| TABLET PC (CHIEFS) | NIC | 172.30.0.2 | 255.255.255.0 | 172.30.0.1 |
| TABLET PC (PIRATES) | NIC | 172.30.1.2 | 255.255.255.128 | 172.30.1.1 |
| TABLET PC (SUNDOWNS) | NIC | 172.30.1.130 | 255.255.255.192 | 172.30.1.129 |
| TABLET PC (SWALLOWS) | NIC | 172.30.1.193 | 255.255.255.192 | 172.30.1.192 |
| Server1 | NIC | 100.10.10.2 | 255.255.255.248 | 100.10.10.1 |
| Server2 | NIC | 30.30.30.2 | 255.255.255.128 | 30.30.30.1 |

Logical Topology

TASK 3

Pc2 Chiefs

Server1

Server2

Smartphone chiefs

Wireless router2 chiefs

Tablet pc pirates

Wireless router pirates

Tablet pc sundowns

Wireless router sundows

Tablet pc swallows

Wireless router swallows

Router 1

Vlan configurations

Router 1

Router#show run

Building configuration...

Current configuration : 2023 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname Router

!

!

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2901/K9 sn FTX1524QQL5-

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

!

interface GigabitEthernet0/0.10

encapsulation dot1Q 10

ip address 172.30.0.5 255.255.255.0

!

interface GigabitEthernet0/0.20

encapsulation dot1Q 20

ip address 172.30.1.5 255.255.255.128

!

interface GigabitEthernet0/0.30

encapsulation dot1Q 30

ip address 172.30.1.135 255.255.255.192

!

interface GigabitEthernet0/0.40

encapsulation dot1Q 40

ip address 172.30.1.195 255.255.255.192

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

ip address 172.30.2.1 255.255.255.252

!

interface Serial0/0/1

ip address 172.30.1.1 255.255.255.128

clock rate 2000000

shutdown

!

interface FastEthernet0/1/0

switchport mode access

switchport nonegotiate

!

interface FastEthernet0/1/1

switchport mode access

switchport nonegotiate

!

interface FastEthernet0/1/2

switchport mode access

switchport nonegotiate

!

interface FastEthernet0/1/3

switchport mode access

switchport nonegotiate

!

interface Serial0/2/0

ip address 172.30.2.5 255.255.255.252

clock rate 2000000

!

interface Serial0/2/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/3/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/3/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

log-adjacency-changes

network 172.30.0.0 0.0.0.255 area 3

network 172.30.1.0 0.0.0.127 area 3

network 172.30.1.192 0.0.0.63 area 3

network 172.30.1.128 0.0.0.63 area 3

network 172.30.2.0 0.0.0.3 area 3

network 172.30.2.4 0.0.0.3 area 3

!

ip classless

!

ip flow-export version 9

!

!

!

no cdp run

!

!

!

!

!

!

line con 0

!

line aux 0

!

line vty 0 4

login

!

!

!

End

Router 2